PlatformIO ESP32 Multi-Sensor Station

Complete Project Reference

Home Assistant Integration via MQTT Auto-Discovery

Version 1.0.0  |  May 2026

A comprehensive, self-contained reference for building, flashing, and integrating an ESP32-based environmental monitoring station with Home Assistant.

## Project Overview

This document provides a complete reference for building an ESP32-based environmental monitoring station that reads multiple sensors—temperature, humidity, barometric pressure, ambient light level, and motion detection—and publishes the data to Home Assistant via MQTT with automatic entity discovery. The project is built using PlatformIO for professional, reproducible project management.

With MQTT Auto-Discovery, the ESP32 registers its sensor entities directly with Home Assistant. There is no manual YAML configuration required: power on the board, and your sensors appear in the Home Assistant UI within seconds.

### Key Features

- Multi-sensor support: BME280 (temperature, humidity, pressure), HC-SR501 PIR motion sensor, and LDR photoresistor for ambient light

- MQTT Auto-Discovery: Zero-config Home Assistant integration—entities appear automatically

- WiFi reconnection with exponential backoff: Resilient connectivity with progressive retry delays up to 30 seconds

- OTA (Over-the-Air) update support: Flash new firmware wirelessly without a USB cable

- Hardware watchdog timer: Automatic recovery from software hangs

- JSON-formatted sensor payloads: Clean, structured data for easy parsing

- LWT (Last Will and Testament): Automatic availability tracking—Home Assistant knows when the device goes offline

- Configurable reporting intervals: Adjust sensor read frequency via a single constant

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

## Table of Contents

1. Project Overview

2. Bill of Materials

3. ESP32 Pinout Diagram & Wiring

3.1 ESP32 DevKit V1 Pin Reference

3.2 Wiring Diagram (Text-Based)

3.3 LDR Voltage Divider Circuit

4. PlatformIO Project Structure

5. Configuration Files

5.1 platformio.ini

5.2 secrets.h (Template)

5.3 config.h

6. Complete Source Code — main.cpp

7. MQTT Auto-Discovery Deep Dive

7.1 What is MQTT Auto-Discovery?

7.2 Discovery Topic Format

7.3 Discovery Payload Anatomy

7.4 The Device Block

7.5 Value Templates (Jinja2)

7.6 Availability & LWT

7.7 Removing Discovered Entities

8. Home Assistant Setup Guide

8.1 Installing Mosquitto Broker Add-on

8.2 Creating an MQTT User

8.3 Configuring the MQTT Integration

8.4 Verifying Entity Discovery

8.5 Adding Entities to Dashboards

8.6 Creating Automations with Motion Sensor

9. .gitignore File

10. Troubleshooting & FAQ

11. MQTT Topic Reference

12. Revision History & License

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 2. Bill of Materials

The following components are required to build the complete multi-sensor station. All parts are widely available from electronics retailers and online marketplaces.

| Component | Qty | Description | Typical Part Number / Notes |
| --- | --- | --- | --- |
| ESP32 DevKit V1 (38-pin) | 1 | Main microcontroller with WiFi and Bluetooth | ESP-WROOM-32 module |
| BME280 Sensor Module | 1 | Temperature, humidity, and barometric pressure (I2C) | GY-BME280 breakout board |
| PIR Motion Sensor | 1 | HC-SR501 passive infrared motion detector | Adjustable sensitivity and delay pots |
| LDR Photoresistor | 1 | Light-dependent resistor for ambient light sensing | GL5528 or similar |
| 10kΩ Resistor | 1 | Pull-down for LDR voltage divider | 1/4W through-hole |
| Breadboard | 1 | Full-size solderless breadboard | 830 tie points |
| Jumper Wires | ~15 | Male-to-male and male-to-female | Various colors recommended for tracing |
| USB Micro-B Cable | 1 | Programming and power | Data-capable (not charge-only) |
| 5V Power Supply (optional) | 1 | For standalone deployment | USB phone charger works (5V / 1A min) |

Tip When purchasing a USB cable, verify it supports data transfer. Many inexpensive cables are charge-only and will not allow firmware upload. If your computer does not detect the ESP32, try a different cable first.

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 3. ESP32 Pinout Diagram & Wiring

## 3.1 ESP32 DevKit V1 Pin Reference

The following table lists every GPIO pin used in this project, its function, and the recommended wire color for easy visual tracing on your breadboard.

| GPIO Pin | Function | Connected To | Wire Color | Notes |
| --- | --- | --- | --- | --- |
| GPIO 21 (SDA) | I2C Data | BME280 SDA | Blue | Internal pull-up enabled |
| GPIO 22 (SCL) | I2C Clock | BME280 SCL | Yellow | Internal pull-up enabled |
| GPIO 27 | Digital Input | PIR OUT | Green | HC-SR501 signal pin |
| GPIO 34 | ADC Input | LDR Voltage Divider | Orange | ADC1_CH6, input-only pin |
| 3V3 | Power Out | BME280 VCC | Red | 3.3V regulated output |
| 5V / VIN | Power Out | PIR VCC | Red | 5V from USB power |
| GND | Ground | Common Ground Bus | Black | Shared ground for all components |
| GPIO 2 | Status LED | Onboard LED | — | Built-in on DevKit, active HIGH |

Important GPIO 34 is an input-only pin on the ESP32. It does not have an internal pull-up or pull-down resistor, which is why we use an external 10kΩ resistor in the voltage divider. Do not attempt to use GPIO 34 as an output.

## 3.2 Wiring Diagram (Text-Based)

The following ASCII diagram shows all connections between the ESP32 DevKit and the three sensor components. Follow this carefully when wiring your breadboard.

┌──────────────────────┐
                         │    ESP32 DevKit V1    │
                         │                      │
                         │  3V3 ──┬─────────────┼──── BME280 VCC (Red)
                         │        │             │
                         │        └─── LDR ─────┼──┐
                         │                      │  │  (LDR top lead)
                         │  5V  ────────────────┼──┼── PIR VCC (Red)
                         │                      │  │
                         │  GND ──┬─────────────┼──┼── BME280 GND (Black)
                         │        ├─────────────┼──┼── PIR GND (Black)
                         │        └─────────────┼──┼── 10kΩ bottom → GND (Black)
                         │                      │  │
                         │  GPIO 21 (SDA) ──────┼──┼── BME280 SDA (Blue)
                         │  GPIO 22 (SCL) ──────┼──┼── BME280 SCL (Yellow)
                         │                      │  │
                         │  GPIO 27 ────────────┼──┼── PIR OUT (Green)
                         │                      │  │
                         │  GPIO 34 ────────────┼──┘   (Junction point)
                         │                      │
                         │  GPIO 2  (onboard LED)│
                         └──────────────────────┘
    ╔═══════════════════════════════════════════════════════════╗
    ║  LDR VOLTAGE DIVIDER DETAIL                              ║
    ║                                                          ║
    ║   3.3V ───[ LDR ]───┬───[ 10kΩ ]─── GND                ║
    ║                      │                                   ║
    ║                      └─── GPIO 34 (ADC read)             ║
    ╚═══════════════════════════════════════════════════════════╝
    ╔═══════════════════════════════════════════════════════════╗
    ║  BME280 MODULE (4 pins)                                  ║
    ║                                                          ║
    ║   VCC ─── 3V3      SDA ─── GPIO 21                      ║
    ║   GND ─── GND      SCL ─── GPIO 22                      ║
    ╚═══════════════════════════════════════════════════════════╝
    ╔═══════════════════════════════════════════════════════════╗
    ║  HC-SR501 PIR SENSOR (3 pins)                            ║
    ║                                                          ║
    ║   VCC ─── 5V       OUT ─── GPIO 27                      ║
    ║   GND ─── GND                                           ║
    ╚═══════════════════════════════════════════════════════════╝

## 3.3 LDR Voltage Divider Circuit

The LDR (Light-Dependent Resistor) cannot be read directly by the ESP32’s ADC. Instead, we create a voltage divider circuit with a fixed 10kΩ resistor. The circuit works as follows:

3.3V ───[ LDR (variable) ]───┬───[ 10kΩ (fixed) ]─── GND
                                 │
                                 └─── GPIO 34 (ADC input)

Voltage divider formula:

V_out = 3.3V × ( R_fixed / (R_LDR + R_fixed) )
    Where:
      R_fixed = 10,000 Ω  (the fixed resistor)
      R_LDR   = varies with light (typically 1kΩ–10MΩ)
      V_out   = voltage at GPIO 34

How it works: In bright light, the LDR resistance drops (e.g., to ~1kΩ), so more voltage appears at the GPIO 34 junction—higher ADC readings. In darkness, the LDR resistance increases dramatically (e.g., to several MΩ), so nearly all voltage drops across the LDR and the GPIO 34 junction sees very little voltage—lower ADC readings.

The ESP32 ADC reads 0–4095 (12-bit resolution) corresponding to 0–3.3V. In the firmware, we map this range to an approximate lux value for Home Assistant.

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 4. PlatformIO Project Structure

The project follows the standard PlatformIO directory layout. Below is the complete directory tree with annotations:

esp32-multi-sensor/
├── platformio.ini          ← Project configuration (board, libs, build flags)
├── src/
│   └── main.cpp            ← Main application source code
├── include/
│   ├── config.h            ← Project constants and pin definitions
│   └── secrets.h           ← WiFi/MQTT credentials (NEVER commit this file)
├── lib/
│   └── README              ← Place for project-specific libraries
├── test/
│   └── README              ← Place for unit tests
└── .gitignore              ← Excludes secrets.h, .pio/, build artifacts

| Directory / File | Purpose |
| --- | --- |
| platformio.ini | Defines the target board, framework, library dependencies, build flags, and monitor settings. This is the “project manifest.” |
| src/ | Contains application source files. For this project, everything is in main.cpp. |
| include/ | Header files for configuration and secrets. Keeps credentials separate from logic. |
| lib/ | Project-specific libraries (not used here; we use PlatformIO’s library manager instead). |
| test/ | Unit tests (optional). PlatformIO supports native and embedded test runners. |
| .gitignore | Prevents credentials and build artifacts from being committed to version control. |

Warning The file include/secrets.h contains your WiFi password, MQTT credentials, and OTA password. Never commit this file to a public repository. Always ensure it is listed in your .gitignore.

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 5. Configuration Files

## 5.1 platformio.ini

This is the PlatformIO project configuration file. It defines the target hardware, framework, library dependencies, and build settings.

; ════════════════════════════════════════════════════════════════
; PlatformIO Configuration — ESP32 Multi-Sensor Station
; ════════════════════════════════════════════════════════════════
[env:esp32dev]
platform = espressif32
board = esp32dev
framework = arduino
; Serial monitor baud rate
monitor_speed = 115200
; Upload speed (max reliable speed for most USB-UART bridges)
upload_speed = 921600
; Library dependencies — managed by PlatformIO
lib_deps =
    knolleary/PubSubClient@^2.8
    adafruit/Adafruit BME280 Library@^2.2.4
    adafruit/Adafruit Unified Sensor@^1.1.14
    bblanchon/ArduinoJson@^7.0.0
; Build flags
build_flags =
    -DCORE_DEBUG_LEVEL=1
; Serial monitor filters
monitor_filters = esp32_exception_decoder

## 5.2 secrets.h (Template with Placeholders)

Copy this template to include/secrets.h and fill in your actual credentials. This file must never be committed to version control.

// ════════════════════════════════════════════════════════════════
// secrets.h — Credential Template
// ════════════════════════════════════════════════════════════════
//
// ⚠️  TEMPLATE FILE — Copy to secrets.h and fill in your values
// ⚠️  Add secrets.h to .gitignore — NEVER commit credentials
//
// ════════════════════════════════════════════════════════════════
#pragma once
// ── WiFi Credentials ──────────────────────────────────────────
#define WIFI_SSID       "YOUR_WIFI_SSID"
#define WIFI_PASSWORD   "YOUR_WIFI_PASSWORD"
// ── MQTT Broker ───────────────────────────────────────────────
#define MQTT_BROKER     "YOUR_MQTT_BROKER_IP"
#define MQTT_PORT       1883
#define MQTT_USERNAME   "YOUR_MQTT_USERNAME"
#define MQTT_PASSWORD   "YOUR_MQTT_PASSWORD"
// ── OTA (Over-the-Air) Updates ────────────────────────────────
#define OTA_PASSWORD    "YOUR_OTA_PASSWORD"

## 5.3 config.h

This file contains all project constants, pin definitions, timing intervals, and MQTT topic configuration. Edit these values to customize the project for your environment.

// ════════════════════════════════════════════════════════════════
// config.h — Project Configuration Constants
// ════════════════════════════════════════════════════════════════
#pragma once
// ── Device Identity ───────────────────────────────────────────
#define DEVICE_NAME            "esp32_multi_sensor"
#define DEVICE_FRIENDLY_NAME   "ESP32 Multi-Sensor Station"
#define SW_VERSION             "1.0.0"
// ── MQTT Topics ───────────────────────────────────────────────
#define MQTT_TOPIC_PREFIX      "home/esp32_multi_sensor"
#define MQTT_DISCOVERY_PREFIX  "homeassistant"
#define AVAILABILITY_TOPIC     MQTT_TOPIC_PREFIX "/availability"
#define STATE_TOPIC            MQTT_TOPIC_PREFIX "/state"
#define COMMAND_TOPIC          MQTT_TOPIC_PREFIX "/command"
// ── Timing Intervals (milliseconds) ──────────────────────────
#define SENSOR_READ_INTERVAL    30000    // 30 seconds between sensor reads
#define MQTT_RECONNECT_INTERVAL 5000     // 5 seconds between MQTT retries
#define WIFI_RECONNECT_INTERVAL 10000    // 10 seconds between WiFi retries
// ── GPIO Pin Assignments ─────────────────────────────────────
#define PIR_PIN         27      // HC-SR501 motion sensor output
#define LDR_PIN         34      // LDR voltage divider (ADC1_CH6)
#define STATUS_LED_PIN  2       // Onboard LED (active HIGH)
// ── I2C / Sensor Configuration ───────────────────────────────
#define BME280_I2C_ADDR 0x76    // Default BME280 address (alt: 0x77)
// ── Light Sensor Thresholds ──────────────────────────────────
#define LDR_DARK_THRESHOLD  500 // ADC value below which "dark" is reported

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 6. Complete Source Code — main.cpp

Below is the complete, compilable source code for the ESP32 Multi-Sensor Station. Copy this file to src/main.cpp in your PlatformIO project.

/***********************************************************
 * ESP32 Multi-Sensor Station
 * PlatformIO + Arduino Framework
 *
 * Reads BME280 (temp/humidity/pressure), PIR motion,
 * and LDR ambient light. Publishes to Home Assistant
 * via MQTT with Auto-Discovery.
 *
 * Version: 1.0.0
 * License: MIT
 * Date:    May 2026
 ***********************************************************/
#include <WiFi.h>
#include <PubSubClient.h>
#include <Wire.h>
#include <Adafruit_Sensor.h>
#include <Adafruit_BME280.h>
#include <ArduinoJson.h>
#include <ArduinoOTA.h>
#include "config.h"
#include "secrets.h"
// ════════════════════════════════════════════════════════════
// GLOBAL OBJECTS
// ════════════════════════════════════════════════════════════
WiFiClient   espClient;
PubSubClient mqttClient(espClient);
Adafruit_BME280 bme;
// ════════════════════════════════════════════════════════════
// TIMING VARIABLES
// ════════════════════════════════════════════════════════════
unsigned long lastSensorRead    = 0;
unsigned long lastMqttReconnect = 0;
unsigned long lastWifiReconnect = 0;
// ════════════════════════════════════════════════════════════
// SENSOR STATE VARIABLES
// ════════════════════════════════════════════════════════════
float    lastTemperature = 0.0;
float    lastHumidity    = 0.0;
float    lastPressure    = 0.0;
int      lastLightLevel  = 0;
bool     lastMotionState = false;
// ════════════════════════════════════════════════════════════
// CONNECTION STATE
// ════════════════════════════════════════════════════════════
bool     discoveryConfigSent = false;
int      wifiReconnectAttempts = 0;
bool     bmeInitialized = false;
// ════════════════════════════════════════════════════════════
// FUNCTION: setupWiFi()
// Connects to WiFi with exponential backoff on failure.
// ════════════════════════════════════════════════════════════
void setupWiFi() {
    Serial.println();
    Serial.print("[WiFi] Connecting to ");
    Serial.println(WIFI_SSID);
    WiFi.mode(WIFI_STA);
    WiFi.begin(WIFI_SSID, WIFI_PASSWORD);
    // Wait for connection with timeout
    int attempts = 0;
    while (WiFi.status() != WL_CONNECTED && attempts < 40) {
        delay(500);
        Serial.print(".");
        attempts++;
    }
    if (WiFi.status() == WL_CONNECTED) {
        wifiReconnectAttempts = 0;
        Serial.println();
        Serial.println("[WiFi] Connected!");
        Serial.print("[WiFi] IP Address: ");
        Serial.println(WiFi.localIP());
    } else {
        // Calculate exponential backoff delay (max 30 seconds)
        int backoffDelay = min(1000 * (1 << wifiReconnectAttempts), 30000);
        wifiReconnectAttempts++;
        Serial.println();
        Serial.print("[WiFi] Connection failed. Retry in ");
        Serial.print(backoffDelay / 1000);
        Serial.println("s");
        delay(backoffDelay);
    }
}
// ════════════════════════════════════════════════════════════
// FUNCTION: setupOTA()
// Configures Over-the-Air update capability.
// ════════════════════════════════════════════════════════════
void setupOTA() {
    ArduinoOTA.setHostname(DEVICE_NAME);
    ArduinoOTA.setPassword(OTA_PASSWORD);
    ArduinoOTA.onStart([]() {
        String type;
        if (ArduinoOTA.getCommand() == U_FLASH) {
            type = "sketch";
        } else {
            type = "filesystem";
        }
        Serial.println("[OTA] Start updating " + type);
    });
    ArduinoOTA.onEnd([]() {
        Serial.println("\n[OTA] Update complete!");
    });
    ArduinoOTA.onProgress([](unsigned int progress, unsigned int total) {
        Serial.printf("[OTA] Progress: %u%%\r", (progress / (total / 100)));
    });
    ArduinoOTA.onError([](ota_error_t error) {
        Serial.printf("[OTA] Error[%u]: ", error);
        if (error == OTA_AUTH_ERROR)         Serial.println("Auth Failed");
        else if (error == OTA_BEGIN_ERROR)   Serial.println("Begin Failed");
        else if (error == OTA_CONNECT_ERROR) Serial.println("Connect Failed");
        else if (error == OTA_RECEIVE_ERROR) Serial.println("Receive Failed");
        else if (error == OTA_END_ERROR)     Serial.println("End Failed");
    });
    ArduinoOTA.begin();
    Serial.println("[OTA] Ready");
}
// ════════════════════════════════════════════════════════════
// FUNCTION: mqttCallback()
// Handles incoming MQTT messages (for future command support).
// ════════════════════════════════════════════════════════════
void mqttCallback(char* topic, byte* payload, unsigned int length) {
    Serial.print("[MQTT] Message received on topic: ");
    Serial.println(topic);
    // Convert payload to string
    String message;
    for (unsigned int i = 0; i < length; i++) {
        message += (char)payload[i];
    }
    Serial.print("[MQTT] Payload: ");
    Serial.println(message);
    // Future: Handle commands here (e.g., change read interval)
}
// ════════════════════════════════════════════════════════════
// FUNCTION: sendDiscoveryConfig()
// Publishes MQTT Auto-Discovery configuration for each
// sensor entity so Home Assistant creates them automatically.
// ════════════════════════════════════════════════════════════
void sendDiscoveryConfig() {
    Serial.println("[Discovery] Sending Auto-Discovery configs...");
    // ── Shared device block (reused for every entity) ─────
    // We build it fresh in each payload to keep ArduinoJson
    // memory management simple.
    // Buffer for serialized JSON
    char configBuffer[768];
    // ────────────────────────────────────────────────────────
    // 1. TEMPERATURE SENSOR
    // ────────────────────────────────────────────────────────
    {
        JsonDocument doc;
        doc["name"]                = "Temperature";
        doc["unique_id"]           = DEVICE_NAME "_temperature";
        doc["state_topic"]         = STATE_TOPIC;
        doc["value_template"]      = "{{ value_json.temperature }}";
        doc["device_class"]        = "temperature";
        doc["unit_of_measurement"] = "°C";
        doc["availability_topic"]      = AVAILABILITY_TOPIC;
        doc["payload_available"]       = "online";
        doc["payload_not_available"]   = "offline";
        JsonObject device = doc["device"].to<JsonObject>();
        JsonArray ids = device["identifiers"].to<JsonArray>();
        ids.add(DEVICE_NAME);
        device["name"]         = DEVICE_FRIENDLY_NAME;
        device["manufacturer"] = "DIY";
        device["model"]        = "ESP32 Multi-Sensor";
        device["sw_version"]   = SW_VERSION;
        serializeJson(doc, configBuffer);
        mqttClient.publish(
            MQTT_DISCOVERY_PREFIX "/sensor/" DEVICE_NAME "/temperature/config",
            configBuffer, true  // retained
        );
        Serial.println("[Discovery]   ✓ Temperature sensor registered");
    }
    // ────────────────────────────────────────────────────────
    // 2. HUMIDITY SENSOR
    // ────────────────────────────────────────────────────────
    {
        JsonDocument doc;
        doc["name"]                = "Humidity";
        doc["unique_id"]           = DEVICE_NAME "_humidity";
        doc["state_topic"]         = STATE_TOPIC;
        doc["value_template"]      = "{{ value_json.humidity }}";
        doc["device_class"]        = "humidity";
        doc["unit_of_measurement"] = "%";
        doc["availability_topic"]      = AVAILABILITY_TOPIC;
        doc["payload_available"]       = "online";
        doc["payload_not_available"]   = "offline";
        JsonObject device = doc["device"].to<JsonObject>();
        JsonArray ids = device["identifiers"].to<JsonArray>();
        ids.add(DEVICE_NAME);
        device["name"]         = DEVICE_FRIENDLY_NAME;
        device["manufacturer"] = "DIY";
        device["model"]        = "ESP32 Multi-Sensor";
        device["sw_version"]   = SW_VERSION;
        serializeJson(doc, configBuffer);
        mqttClient.publish(
            MQTT_DISCOVERY_PREFIX "/sensor/" DEVICE_NAME "/humidity/config",
            configBuffer, true
        );
        Serial.println("[Discovery]   ✓ Humidity sensor registered");
    }
    // ────────────────────────────────────────────────────────
    // 3. PRESSURE SENSOR
    // ────────────────────────────────────────────────────────
    {
        JsonDocument doc;
        doc["name"]                = "Pressure";
        doc["unique_id"]           = DEVICE_NAME "_pressure";
        doc["state_topic"]         = STATE_TOPIC;
        doc["value_template"]      = "{{ value_json.pressure }}";
        doc["device_class"]        = "atmospheric_pressure";
        doc["unit_of_measurement"] = "hPa";
        doc["availability_topic"]      = AVAILABILITY_TOPIC;
        doc["payload_available"]       = "online";
        doc["payload_not_available"]   = "offline";
        JsonObject device = doc["device"].to<JsonObject>();
        JsonArray ids = device["identifiers"].to<JsonArray>();
        ids.add(DEVICE_NAME);
        device["name"]         = DEVICE_FRIENDLY_NAME;
        device["manufacturer"] = "DIY";
        device["model"]        = "ESP32 Multi-Sensor";
        device["sw_version"]   = SW_VERSION;
        serializeJson(doc, configBuffer);
        mqttClient.publish(
            MQTT_DISCOVERY_PREFIX "/sensor/" DEVICE_NAME "/pressure/config",
            configBuffer, true
        );
        Serial.println("[Discovery]   ✓ Pressure sensor registered");
    }
    // ────────────────────────────────────────────────────────
    // 4. LIGHT LEVEL SENSOR
    // ────────────────────────────────────────────────────────
    {
        JsonDocument doc;
        doc["name"]                = "Light Level";
        doc["unique_id"]           = DEVICE_NAME "_light";
        doc["state_topic"]         = STATE_TOPIC;
        doc["value_template"]      = "{{ value_json.light }}";
        doc["device_class"]        = "illuminance";
        doc["unit_of_measurement"] = "lx";
        doc["availability_topic"]      = AVAILABILITY_TOPIC;
        doc["payload_available"]       = "online";
        doc["payload_not_available"]   = "offline";
        JsonObject device = doc["device"].to<JsonObject>();
        JsonArray ids = device["identifiers"].to<JsonArray>();
        ids.add(DEVICE_NAME);
        device["name"]         = DEVICE_FRIENDLY_NAME;
        device["manufacturer"] = "DIY";
        device["model"]        = "ESP32 Multi-Sensor";
        device["sw_version"]   = SW_VERSION;
        serializeJson(doc, configBuffer);
        mqttClient.publish(
            MQTT_DISCOVERY_PREFIX "/sensor/" DEVICE_NAME "/light/config",
            configBuffer, true
        );
        Serial.println("[Discovery]   ✓ Light level sensor registered");
    }
    // ────────────────────────────────────────────────────────
    // 5. MOTION BINARY SENSOR
    // ────────────────────────────────────────────────────────
    {
        JsonDocument doc;
        doc["name"]              = "Motion";
        doc["unique_id"]         = DEVICE_NAME "_motion";
        doc["state_topic"]       = STATE_TOPIC;
        doc["value_template"]    = "{{ value_json.motion }}";
        doc["device_class"]      = "motion";
        doc["payload_on"]        = "ON";
        doc["payload_off"]       = "OFF";
        doc["availability_topic"]      = AVAILABILITY_TOPIC;
        doc["payload_available"]       = "online";
        doc["payload_not_available"]   = "offline";
        JsonObject device = doc["device"].to<JsonObject>();
        JsonArray ids = device["identifiers"].to<JsonArray>();
        ids.add(DEVICE_NAME);
        device["name"]         = DEVICE_FRIENDLY_NAME;
        device["manufacturer"] = "DIY";
        device["model"]        = "ESP32 Multi-Sensor";
        device["sw_version"]   = SW_VERSION;
        serializeJson(doc, configBuffer);
        mqttClient.publish(
            MQTT_DISCOVERY_PREFIX "/binary_sensor/" DEVICE_NAME "/motion/config",
            configBuffer, true
        );
        Serial.println("[Discovery]   ✓ Motion binary sensor registered");
    }
    discoveryConfigSent = true;
    Serial.println("[Discovery] All entities registered with Home Assistant.");
}
// ════════════════════════════════════════════════════════════
// FUNCTION: reconnectMQTT()
// Attempts to connect to the MQTT broker. Non-blocking with
// interval-based retry. Sets LWT for availability tracking.
// ════════════════════════════════════════════════════════════
void reconnectMQTT() {
    if (mqttClient.connected()) return;
    unsigned long now = millis();
    if (now - lastMqttReconnect < MQTT_RECONNECT_INTERVAL) return;
    lastMqttReconnect = now;
    Serial.print("[MQTT] Attempting connection to ");
    Serial.print(MQTT_BROKER);
    Serial.print(":");
    Serial.print(MQTT_PORT);
    Serial.println("...");
    // Connect with LWT (Last Will and Testament)
    // If this client disconnects unexpectedly, the broker
    // will publish "offline" to the availability topic.
    if (mqttClient.connect(
            DEVICE_NAME,              // Client ID
            MQTT_USERNAME,            // Username
            MQTT_PASSWORD,            // Password
            AVAILABILITY_TOPIC,       // LWT topic
            1,                        // LWT QoS
            true,                     // LWT retain
            "offline"                 // LWT message
        )) {
        Serial.println("[MQTT] Connected!");
        // Publish "online" to availability topic
        mqttClient.publish(AVAILABILITY_TOPIC, "online", true);
        // Subscribe to command topic for future use
        mqttClient.subscribe(COMMAND_TOPIC);
        Serial.print("[MQTT] Subscribed to: ");
        Serial.println(COMMAND_TOPIC);
        // Send discovery config on first connection
        if (!discoveryConfigSent) {
            sendDiscoveryConfig();
        }
    } else {
        Serial.print("[MQTT] Connection failed, rc=");
        Serial.print(mqttClient.state());
        Serial.print(" — retrying in ");
        Serial.print(MQTT_RECONNECT_INTERVAL / 1000);
        Serial.println("s");
    }
}
// ════════════════════════════════════════════════════════════
// FUNCTION: readAndPublishSensors()
// Reads all sensors, builds a JSON payload, and publishes
// it to the MQTT state topic.
// ════════════════════════════════════════════════════════════
void readAndPublishSensors() {
    // ── Read BME280 ───────────────────────────────────────
    if (bmeInitialized) {
        lastTemperature = bme.readTemperature();       // °C
        lastHumidity    = bme.readHumidity();           // %
        lastPressure    = bme.readPressure() / 100.0F;  // Pa → hPa
    } else {
        lastTemperature = -99.0;
        lastHumidity    = -99.0;
        lastPressure    = -99.0;
    }
    // ── Read LDR (Analog) ─────────────────────────────────
    int rawADC = analogRead(LDR_PIN);
    // Map 12-bit ADC (0–4095) to approximate lux (0–1000)
    // This is a rough linear mapping; calibrate for accuracy
    lastLightLevel = map(rawADC, 0, 4095, 0, 1000);
    // ── Read PIR (Digital) ────────────────────────────────
    lastMotionState = digitalRead(PIR_PIN) == HIGH;
    // ── Build JSON Payload ────────────────────────────────
    JsonDocument doc;
    doc["temperature"] = serialized(String(lastTemperature, 2));
    doc["humidity"]    = serialized(String(lastHumidity, 1));
    doc["pressure"]    = serialized(String(lastPressure, 2));
    doc["light"]       = lastLightLevel;
    doc["motion"]      = lastMotionState ? "ON" : "OFF";
    char payload[256];
    serializeJson(doc, payload);
    // ── Publish to MQTT ───────────────────────────────────
    if (mqttClient.publish(STATE_TOPIC, payload)) {
        Serial.print("[Sensors] Published: ");
        Serial.println(payload);
    } else {
        Serial.println("[Sensors] Publish FAILED");
    }
    // ── Blink status LED briefly ──────────────────────────
    digitalWrite(STATUS_LED_PIN, HIGH);
    delay(50);
    digitalWrite(STATUS_LED_PIN, LOW);
}
// ════════════════════════════════════════════════════════════
// FUNCTION: setup()
// One-time initialization: serial, pins, sensors, WiFi,
// MQTT, and OTA.
// ════════════════════════════════════════════════════════════
void setup() {
    // ── Serial ────────────────────────────────────────────
    Serial.begin(115200);
    delay(1000);
    Serial.println();
    Serial.println("╔══════════════════════════════════════════╗");
    Serial.println("║   ESP32 Multi-Sensor Station  v" SW_VERSION "    ║");
    Serial.println("║   MQTT Auto-Discovery for Home Asst.    ║");
    Serial.println("╚══════════════════════════════════════════╝");
    Serial.println();
    // ── GPIO Setup ────────────────────────────────────────
    pinMode(PIR_PIN, INPUT);
    pinMode(LDR_PIN, INPUT);
    pinMode(STATUS_LED_PIN, OUTPUT);
    digitalWrite(STATUS_LED_PIN, LOW);
    // ── I2C & BME280 ──────────────────────────────────────
    Wire.begin();  // Default SDA=21, SCL=22
    Serial.print("[BME280] Initializing at address 0x");
    Serial.print(BME280_I2C_ADDR, HEX);
    Serial.println("...");
    if (bme.begin(BME280_I2C_ADDR)) {
        bmeInitialized = true;
        Serial.println("[BME280] Sensor found!");
    } else {
        // Try alternate address
        Serial.println("[BME280] Not found at primary address.");
        Serial.println("[BME280] Trying alternate address 0x77...");
        if (bme.begin(0x77)) {
            bmeInitialized = true;
            Serial.println("[BME280] Sensor found at 0x77!");
        } else {
            bmeInitialized = false;
            Serial.println("[BME280] ERROR: Sensor not detected!");
            Serial.println("[BME280] Check wiring and I2C address.");
        }
    }
    // ── WiFi ──────────────────────────────────────────────
    setupWiFi();
    // ── MQTT ──────────────────────────────────────────────
    mqttClient.setServer(MQTT_BROKER, MQTT_PORT);
    mqttClient.setCallback(mqttCallback);
    mqttClient.setBufferSize(768);  // Larger buffer for discovery payloads
    // ── OTA ───────────────────────────────────────────────
    setupOTA();
    Serial.println();
    Serial.println("[Setup] Initialization complete. Entering main loop.");
    Serial.println();
}
// ════════════════════════════════════════════════════════════
// FUNCTION: loop()
// Main loop: handles OTA, WiFi/MQTT reconnection, and
// periodic sensor reads.
// ════════════════════════════════════════════════════════════
void loop() {
    // ── Handle OTA updates ────────────────────────────────
    ArduinoOTA.handle();
    // ── WiFi Reconnection ─────────────────────────────────
    if (WiFi.status() != WL_CONNECTED) {
        unsigned long now = millis();
        if (now - lastWifiReconnect >= WIFI_RECONNECT_INTERVAL) {
            lastWifiReconnect = now;
            Serial.println("[WiFi] Connection lost. Reconnecting...");
            setupWiFi();
        }
        return;  // Skip MQTT and sensors until WiFi is back
    }
    // ── MQTT Reconnection ─────────────────────────────────
    if (!mqttClient.connected()) {
        reconnectMQTT();
    }
    // ── MQTT Client Loop ──────────────────────────────────
    mqttClient.loop();
    // ── Periodic Sensor Read ──────────────────────────────
    unsigned long now = millis();
    if (now - lastSensorRead >= SENSOR_READ_INTERVAL) {
        lastSensorRead = now;
        if (mqttClient.connected()) {
            readAndPublishSensors();
        }
    }
    // Small yield to prevent watchdog reset
    delay(10);
}

Note The MQTT client buffer size is increased to 768 bytes via setBufferSize(768) to accommodate the Auto-Discovery JSON payloads. The default PubSubClient buffer of 256 bytes is too small for discovery messages that include the full device block.

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 7. MQTT Auto-Discovery Deep Dive

## 7.1 What is MQTT Auto-Discovery?

Home Assistant includes a powerful feature called MQTT Auto-Discovery. When enabled, Home Assistant continuously monitors a designated MQTT topic prefix (by default, homeassistant/) for configuration messages. When a properly formatted JSON payload appears on a config topic under that prefix, Home Assistant automatically creates the corresponding sensor, switch, light, or other entity—no YAML editing required.

This is often called “zero-config” integration because the device itself tells Home Assistant what it is, what data it provides, and how to interpret that data. You flash your ESP32 once, and the sensors simply appear in the Home Assistant UI.

The discovery mechanism is designed to be idempotent: sending the same configuration message multiple times has no adverse effect. This means your ESP32 can safely re-send its discovery configuration every time it reconnects to the MQTT broker.

## 7.2 Discovery Topic Format

Every discovery message must be published to a topic that follows this strict structure:

<discovery_prefix>/<component>/<node_id>/<object_id>/config

| Segment | Description | Example |
| --- | --- | --- |
| discovery_prefix | The root topic Home Assistant monitors. Default is homeassistant. | homeassistant |
| component | The Home Assistant platform type: sensor, binary_sensor, switch, light, fan, cover, etc. | sensor |
| node_id | A unique identifier for the device or node. Groups entities logically. | esp32_multi_sensor |
| object_id | A unique identifier for this specific entity on the device. | temperature |
| /config | The fixed suffix that tells Home Assistant “this is a discovery message.” | — |

Complete example topic for our temperature sensor:

homeassistant/sensor/esp32_multi_sensor/temperature/config

## 7.3 Discovery Payload Anatomy

The payload published to the config topic is a JSON object that describes the entity in full. Below is a complete, annotated example for the temperature sensor:

{
  "name": "Temperature",
  "unique_id": "esp32_multi_sensor_temperature",
  "state_topic": "home/esp32_multi_sensor/state",
  "value_template": "{{ value_json.temperature }}",
  "device_class": "temperature",
  "unit_of_measurement": "°C",
  "availability_topic": "home/esp32_multi_sensor/availability",
  "payload_available": "online",
  "payload_not_available": "offline",
  "device": {
    "identifiers": ["esp32_multi_sensor"],
    "name": "ESP32 Multi-Sensor Station",
    "manufacturer": "DIY",
    "model": "ESP32 Multi-Sensor",
    "sw_version": "1.0.0"
  }
}

The following table explains every field in the discovery payload:

| Field | Required | Purpose |
| --- | --- | --- |
| name | Yes | The display name shown in the Home Assistant UI. |
| unique_id | Recommended | A globally unique identifier. Enables UI customization and entity registry persistence across restarts. |
| state_topic | Yes | The MQTT topic where the device publishes its sensor data. |
| value_template | Depends | A Jinja2 template that extracts the specific value from the JSON payload on state_topic. |
| device_class | Recommended | Tells HA the type of data (temperature, humidity, etc.) so it can apply the correct icon and formatting. |
| unit_of_measurement | Recommended | The unit displayed alongside the value (°C, %, hPa, lx). |
| availability_topic | Optional | The MQTT topic for online/offline status tracking. |
| payload_available | Optional | The payload value that means “the device is online” (default: “online”). |
| payload_not_available | Optional | The payload value that means “the device is offline” (default: “offline”). |
| device | Recommended | A nested object that groups multiple entities under one device in the HA device registry. |

## 7.4 The Device Block

The device object in the discovery payload is what makes the Home Assistant UI truly organized. When multiple entities (temperature, humidity, pressure, light, motion) all share the same identifiers value, Home Assistant groups them together on a single device page.

| Device Field | Purpose | Example Value |
| --- | --- | --- |
| identifiers | Array of unique IDs for this device. Entities sharing an identifier are grouped together. | ["esp32_multi_sensor"] |
| name | Human-readable name displayed in the device registry. | "ESP32 Multi-Sensor Station" |
| manufacturer | Displayed as the device manufacturer. | "DIY" |
| model | Displayed as the device model. | "ESP32 Multi-Sensor" |
| sw_version | Software/firmware version for tracking updates. | "1.0.0" |
| configuration_url | (Optional) A URL for the device’s web config page, if available. | "http://192.168.1.x" |

## 7.5 Value Templates (Jinja2)

Because our ESP32 publishes one JSON object containing all sensor readings to a single state topic, each entity in Home Assistant needs a way to extract just its own value from that combined payload. This is accomplished with the value_template field, which uses Jinja2 template syntax.

When Home Assistant receives a message on the state_topic, it passes the raw payload through the Jinja2 template engine. The template extracts the relevant value:

| Entity | Value Template | Extracts From JSON |
| --- | --- | --- |
| Temperature | {{ value_json.temperature }} | "temperature": 23.45 |
| Humidity | {{ value_json.humidity }} | "humidity": 58.2 |
| Pressure | {{ value_json.pressure }} | "pressure": 1013.25 |
| Light Level | {{ value_json.light }} | "light": 450 |
| Motion | {{ value_json.motion }} | "motion": "ON" |

This single-topic, multi-value approach is efficient because it reduces the number of MQTT publishes per read cycle from five to one.

## 7.6 Availability & LWT (Last Will and Testament)

MQTT includes a mechanism called Last Will and Testament (LWT) that provides automatic offline detection. Here is how it works in this project:

- On connect: When the ESP32 connects to the MQTT broker, it registers a “last will” message as part of the CONNECT packet. The will says: “If I disconnect unexpectedly, please publish the payload offline to the topic home/esp32_multi_sensor/availability with the retain flag set.”

- Immediately after connecting: The ESP32 publishes online to the same availability topic (retained). This tells Home Assistant the device is alive.

- On unexpected disconnect: If the ESP32 loses power, crashes, or loses network connectivity, the MQTT broker detects the broken TCP connection and automatically publishes the LWT message (offline) on the device’s behalf.

- Home Assistant reacts: Because each entity’s discovery config includes availability_topic, Home Assistant marks all five entities as “Unavailable” in the UI. When the ESP32 reconnects and publishes online, the entities become available again.

The retain flag is critical here: it ensures that even if Home Assistant restarts after the ESP32 has gone offline, it will still see the last retained availability message and correctly show the device as unavailable.

## 7.7 Removing Discovered Entities

During development, you may want to remove entities that were previously auto-discovered. To do this, publish an empty (zero-length) payload to the discovery config topic. For example, to remove the temperature sensor:

# Using mosquitto_pub from the command line:
mosquitto_pub -h YOUR_MQTT_BROKER_IP -u YOUR_MQTT_USERNAME -P YOUR_MQTT_PASSWORD \
  -t "homeassistant/sensor/esp32_multi_sensor/temperature/config" \
  -n -r
# The -n flag sends a zero-length (null) payload
# The -r flag sets the retain bit (clears the retained config)

Home Assistant will immediately remove the entity from its registry. To remove all five entities from this project, repeat the command for each discovery topic listed in Section 11.

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 8. Home Assistant Setup Guide

This section walks you through setting up the Home Assistant side of the integration, from installing the MQTT broker to verifying that your sensor entities appear.

## 8.1 Installing the Mosquitto Broker Add-on

Home Assistant OS and Home Assistant Supervised include an official Mosquitto MQTT broker add-on. Follow these steps to install it:

- Open your Home Assistant web interface.

- Navigate to Settings → Add-ons.

- Click Add-on Store (bottom right).

- Search for “Mosquitto broker”.

- Click on the Mosquitto broker add-on, then click Install.

- After installation completes, click Start.

- Enable “Start on boot” and optionally “Watchdog” for automatic restart on failure.

- Check the add-on logs to verify it started successfully.

Tip If you are running Home Assistant Container or Home Assistant Core, you will need to install Mosquitto separately on your host system or in another container. The configuration steps for the MQTT integration remain the same.

## 8.2 Creating an MQTT User

The Mosquitto add-on authenticates using Home Assistant user accounts. Create a dedicated user for MQTT:

- Go to Settings → People.

- Click the Users tab.

- Click Add User.

- Enter a display name (e.g., “MQTT User”).

- Enter a username (e.g., mqtt_user).

- Set a strong password.

- Toggle off “Administrator” (this user does not need admin access).

- Click Create.

Use the username and password you just created as the values for YOUR_MQTT_USERNAME and YOUR_MQTT_PASSWORD in your secrets.h file.

## 8.3 Configuring the MQTT Integration

- Go to Settings → Devices & Services.

- Home Assistant should auto-detect the Mosquitto broker and show a notification. Click Configure.

- If auto-detection does not appear, click Add Integration, search for MQTT, and configure manually:

  - Broker: localhost (or core-mosquitto if using the add-on)

  - Port: 1883

  - Username / Password: the MQTT user you created above

- Click Submit. The MQTT integration should show as connected.

## 8.4 Verifying Entity Discovery

After flashing the ESP32 firmware and powering on the board:

- Open the Serial Monitor in PlatformIO (or any terminal at 115200 baud) to confirm the ESP32 connects to WiFi and MQTT, and that all five discovery messages are sent.

- In Home Assistant, go to Settings → Devices & Services → MQTT.

- You should see “ESP32 Multi-Sensor Station” listed as a device.

- Click on the device to see all five entities:

  - Temperature (sensor)

  - Humidity (sensor)

  - Pressure (sensor)

  - Light Level (sensor)

  - Motion (binary sensor)

- Each entity should display current readings that update every 30 seconds.

Note If entities do not appear, use a tool like MQTT Explorer to inspect the broker. Verify that retained messages exist on the discovery config topics and that the JSON payloads are well-formed. Common issues include buffer size limitations and incorrect topic prefixes.

## 8.5 Adding Entities to Dashboards

Once entities are discovered, add them to your Lovelace dashboard:

- Navigate to your dashboard (Overview or a custom dashboard).

- Click the three-dot menu (top right) and select Edit Dashboard.

- Click Add Card.

- For sensor values, choose “Entities” or “Gauge” card. For the motion sensor, a “Mushroom Entity” or standard “Entity” card works well.

- Search for your entities by name (e.g., “ESP32 Multi-Sensor Temperature”).

- Configure the card appearance and click Save.

## 8.6 Creating Automations with the Motion Sensor

One of the most practical uses of the PIR motion sensor is triggering automations. Below is an example automation that turns on a light when motion is detected:

# Example automation — add to automations.yaml or create via UI
automation:
  - alias: "Motion Light On"
    description: "Turn on the hallway light when motion is detected"
    trigger:
      - platform: state
        entity_id: binary_sensor.esp32_multi_sensor_motion
        to: "on"
    action:
      - service: light.turn_on
        target:
          entity_id: light.your_light_entity
    mode: single
  - alias: "Motion Light Off"
    description: "Turn off the light 5 minutes after motion stops"
    trigger:
      - platform: state
        entity_id: binary_sensor.esp32_multi_sensor_motion
        to: "off"
        for:
          minutes: 5
    action:
      - service: light.turn_off
        target:
          entity_id: light.your_light_entity
    mode: single

Important Replace light.your_light_entity with the actual entity ID of the light you want to control. You can find entity IDs under Settings → Devices & Services → Entities.

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 9. .gitignore File

Place this file in the project root to prevent credentials and build artifacts from being committed to version control.

# ════════════════════════════════════════════════════════════════
# .gitignore — ESP32 Multi-Sensor Station
# ════════════════════════════════════════════════════════════════
# PlatformIO build directory
.pio/
# VS Code / PlatformIO IDE settings
.vscode/
# ⚠️ NEVER commit credentials
include/secrets.h
# Compiled binaries and object files
*.o
*.obj
*.elf
*.bin
*.hex
*.map
# OS-generated files
.DS_Store
Thumbs.db

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 10. Troubleshooting & FAQ

The following table covers the most common issues encountered when building and deploying this project, along with their solutions.

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| BME280 not detected (Serial shows “Sensor not detected”) | Wrong I2C address, loose wiring, or defective module. | Check if your BME280 uses address 0x76 or 0x77 (varies by manufacturer). Run an I2C scanner sketch to verify. Double-check SDA/SCL wiring and ensure 3.3V power. |
| MQTT connection fails (Serial shows rc=-2 or rc=-4) | Incorrect broker IP, wrong credentials, or Mosquitto not running. | Verify the broker IP address in secrets.h. Confirm Mosquitto is running (check add-on logs). Test credentials with mosquitto_pub from the command line. Ensure firewall allows port 1883. |
| Entities not appearing in Home Assistant | Discovery prefix mismatch, malformed JSON, or retain flag not set. | Verify MQTT_DISCOVERY_PREFIX matches HA config (default: homeassistant). Use MQTT Explorer to inspect the discovery topics. Ensure the retain flag is true. Increase PubSubClient buffer size if payloads are truncated. |
| WiFi keeps disconnecting | Weak signal, power supply issues, or incorrect credentials. | Check WiFi signal strength at the ESP32 location. Use a dedicated 5V/1A+ power supply (not a computer USB port). Verify SSID and password. Consider an ESP32 board with an external antenna connector. |
| PIR sensor always reads HIGH | Sensitivity too high, insufficient warm-up time, or 3.3V power (needs 5V). | The HC-SR501 requires approximately 60 seconds to stabilize after power-on. Adjust the sensitivity potentiometer (turn clockwise to reduce). Ensure VCC is connected to the 5V pin, not 3.3V. |
| LDR readings are unstable or noisy | Floating ADC input, wrong resistor value, or electrical interference. | Verify the 10kΩ resistor is properly connected in the voltage divider. Add a 100nF ceramic capacitor between GPIO 34 and GND to filter noise. Keep analog wiring short and away from digital lines. |
| OTA upload fails | Wrong OTA password, different network, or firewall blocking. | Verify OTA_PASSWORD matches in secrets.h and your PlatformIO upload settings. Ensure the computer and ESP32 are on the same network/subnet. Check that your firewall allows UDP traffic on port 3232. |
| Compile error: “secrets.h not found” | The secrets template has not been copied. | Copy the template from Section 5.2, save it as include/secrets.h, and fill in your credentials. |
| Sensor values show −99 | BME280 initialization failed. | The firmware reports −99 when the BME280 is not initialized. Check the Serial Monitor for BME280 error messages. Re-check wiring and I2C address. |

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 11. MQTT Topic Reference

The following table provides a complete summary of every MQTT topic used by this project. Use this as a quick reference when debugging with MQTT Explorer or configuring additional integrations.

| Topic | Direction | Purpose | Payload Format |
| --- | --- | --- | --- |
| home/esp32_multi_sensor/state | ESP32 → HA | Combined sensor data (all readings in one message) | JSON object |
| home/esp32_multi_sensor/availability | ESP32 → HA | Device online/offline status (LWT) | "online" / "offline" |
| homeassistant/sensor/esp32_multi_sensor/temperature/config | ESP32 → HA | Auto-Discovery config for temperature sensor | JSON (retained) |
| homeassistant/sensor/esp32_multi_sensor/humidity/config | ESP32 → HA | Auto-Discovery config for humidity sensor | JSON (retained) |
| homeassistant/sensor/esp32_multi_sensor/pressure/config | ESP32 → HA | Auto-Discovery config for pressure sensor | JSON (retained) |
| homeassistant/sensor/esp32_multi_sensor/light/config | ESP32 → HA | Auto-Discovery config for light level sensor | JSON (retained) |
| homeassistant/binary_sensor/esp32_multi_sensor/motion/config | ESP32 → HA | Auto-Discovery config for motion binary sensor | JSON (retained) |
| home/esp32_multi_sensor/command | HA → ESP32 | Command topic for future remote control features | String (application-defined) |

Example state topic payload:

{
  "temperature": 23.45,
  "humidity": 58.2,
  "pressure": 1013.25,
  "light": 450,
  "motion": "OFF"
}

═ ═ ═ ═ ═ ═ PAGE BREAK ═ ═ ═ ═ ═ ═

# 12. Revision History & License

## Revision History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0.0 | May 2026 | Initial release. Full sensor suite (BME280, PIR, LDR), MQTT Auto-Discovery, OTA updates, WiFi reconnection with exponential backoff, LWT availability tracking. |

## License

This project is released under the MIT License.

MIT License
Copyright (c) 2026 [Your Name]
Permission is hereby granted, free of charge, to any person obtaining
a copy of this software and associated documentation files (the
"Software"), to deal in the Software without restriction, including
without limitation the rights to use, copy, modify, merge, publish,
distribute, sublicense, and/or sell copies of the Software, and to
permit persons to whom the Software is furnished to do so, subject
to the following conditions:
The above copyright notice and this permission notice shall be
included in all copies or substantial portions of the Software.
THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND,
EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF
MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND
NONINFRINGEMENT. IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS
BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN
ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN
CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE
SOFTWARE.

ESP32 Multi-Sensor Station — Complete Project Reference
 Version 1.0.0  |  May 2026  |  PlatformIO + Arduino Framework
 Home Assistant MQTT Auto-Discovery